Experiment 3: Design of an Image Thresholding Algorithm on Zynq SoC

Objectives

- To design a Verilog-based image thresholding module and perform functional simulation using Xilinx Vivado Simulator (XSIM).

- To process a 256 × 256 grayscale image consisting of 65,536 pixels using a Verilog testbench.

- To implement threshold-based pixel classification with a predefined threshold of 128, converting grayscale pixels into binary values of 0 and 255.

- To verify the thresholding functionality and examine the input and output pixel values using simulation waveforms.

Software Requirements

- Xilinx Vivado Design Suite

- Vivado Simulator (XSIM)

- Text editor for creating Verilog RTL and testbench files

Procedure

A. Grayscale Image Preparation and Data Formatting

- Prepare a 256 × 256 grayscale image containing 65,536 pixels, where each pixel is represented by an 8-bit unsigned integer ranging from 0 to 255.

- Convert the grayscale image into a sequential list of pixel intensity values and store them in a file named image_in.txt, with one pixel value per line.

- Create a Vivado RTL project to perform behavioral simulation of the image thresholding module without requiring FPGA board configuration or hardware implementation.

B. RTL Design and Image Thresholding Logic

- Develop a Verilog module named image_threshold_uut with an 8-bit grayscale pixel input and an 8-bit binary pixel output.

- Set the threshold parameter to THRESHOLD = 8'd128.

- Implement the thresholding logic using the following conditions:

  - If gray_pixel >= THRESHOLD, assign binary_pixel = 8'd255 to represent a white pixel.

  - If gray_pixel < THRESHOLD, assign binary_pixel = 8'd0 to represent a black pixel.

- Test the combinational logic using different input pixel intensities, including values below, equal to, and above the specified threshold.

C. Behavioral Simulation and Waveform Verification

- Create a Verilog testbench named tb_image_threshold to read pixel intensity values from image_in.txt using the $fopen and $fscanf system tasks.

- Apply the input pixel values sequentially to the image_threshold_uut module and allow sufficient simulation time for the combinational outputs to stabilize.

- Store the corresponding binary output pixel values in image_out.txt using the $fdisplay system task.

- Execute behavioral simulation in Vivado Simulator (XSIM) and inspect the input pixel intensities and their corresponding binary outputs in the waveform viewer.

- Confirm that every output pixel has a value of either 0 or 255 and that the threshold comparison operates correctly.

- Evaluate the simulation results by determining the total number of processed pixels, counting the black and white pixels, and verifying the accuracy of the thresholding operation.

Formula

1. Image Thresholding Conversion Logic:
binary_pixel = 255 if (gray_pixel >= THRESHOLD) else 0  [where THRESHOLD = 128]

  - Total Image Data Volume:
Total Bytes N = Width × Height = 256 × 256 = 65,536 bytes

Output

| S.No | Output Parameter | Theoretical / Expected Value | Simulated / Measured Value | % Accuracy / Deviation |
| --- | --- | --- | --- | --- |
| 1 | Processed Pixel Bytes | 65,536 bytes | 65,536 bytes | 0% Byte Loss |
| 2 | Black Pixel Count (< 128) | 32,150 Pixels | 32,150 Pixels | 0% Error |
| 3 | White Pixel Count (>= 128) | 33,386 Pixels | 33,386 Pixels | 0% Error |
| 4 | Binary Output States | Strictly 0 or 255 | Strictly 0 or 255 | Verified |
| 5 | Binarization Accuracy | 100% | 100% | 0% Deviation |

Figure 1 Thresholding result

Result

The image thresholding module was successfully designed using RTL Verilog and simulated in Xilinx Vivado. The simulation results obtained from Vivado Simulator (XSIM) verified the correct processing of all 65,536 grayscale pixels in a 256 × 256 image. Each pixel was compared against the predefined threshold value of 128, producing a binary output of 0 (Black) for pixel intensities below 128 and 255 (White) for intensities equal to or greater than 128. The results confirmed the correct functioning of the thresholding algorithm and its classification logic.