Experiment 4: Design of an Image Intensity Inversion Circuit on Zynq SoC

Objectives

- To design a Verilog-based image intensity inversion module and perform functional simulation using Xilinx Vivado Simulator (XSIM).

- To process a 256 × 256 grayscale image consisting of 65,536 pixels using a Verilog testbench.

- To implement 8-bit grayscale intensity inversion using the mathematical expression (P_{out} = 255 - P_{in}).

- To verify the inverted pixel values and analyze the input and output signals using Vivado simulation waveforms.

Software Requirements

- Xilinx Vivado Design Suite

- Vivado Simulator (XSIM)

- Text editor for developing Verilog RTL and testbench files

Procedure

A. Grayscale Image Preparation and Data Formatting

- Prepare a 256 × 256 grayscale image containing 65,536 pixels, with each pixel represented by an 8-bit unsigned integer ranging from 0 to 255.

- Convert the image into a sequential list of pixel intensity values and store them in a file named image_in.txt, with one pixel value per line.

- Create a Vivado RTL project to perform behavioral simulation of the image intensity inversion module without requiring FPGA board configuration or physical hardware implementation.

B. RTL Design and Intensity Inversion Logic

- Develop a Verilog module named intensity_inversion with an 8-bit pixel input and an 8-bit pixel output.

- Implement the intensity inversion operation using the following equation:

pixel_out = 8'd255 - pixel_in

- If synchronous operation is required, update the output at the rising edge of the clock when valid_in is asserted. Incorporate reset and valid_out signals if required by the design specification.

- Verify the inversion functionality using representative input values and their corresponding outputs:

  - Input 0 → Output 255

  - Input 100 → Output 155

  - Input 128 → Output 127

  - Input 255 → Output 0

C. Behavioral Simulation and Waveform Verification

- Develop a Verilog testbench named tb_intensity_inversion to read pixel intensity values from image_in.txt using the $fopen and $fscanf system tasks.

- Apply the pixel values sequentially to the intensity inversion module and allow sufficient simulation time for the corresponding outputs to be generated.

- Save the inverted pixel values in a file named image_out.txt using the $fdisplay system task.

- Run behavioral simulation using Vivado Simulator (XSIM) and examine the input pixels, clock, reset, valid signals (if applicable), and output pixels in the waveform viewer.

- Verify that each output pixel satisfies the intensity inversion equation (P_{out} = 255 - P_{in}).

- Confirm the correct processing of all 65,536 pixels and document the simulation results using representative input-output values and waveform observations.

- Formulas

- 1. Image Intensity Inversion Relationship: P_out = 255 - P_in

- 2. Total Data Volume for 256×256 Image: N = 256 x 256 = 65,536

Output

| S.No | Output Parameter | Expected / Calculated Output | Simulated / Measured Output | Result / Deviation |
| --- | --- | --- | --- | --- |
| 1 | Processed Frame Size | 65,536 Bytes | 65,536 Bytes | 0% Byte Loss |
| 2 | Sample Inversion (Input: 0) | P_out = 255 | P_out = 255 | 0% Error |
| 3 | Sample Inversion (Input: 100) | P_out = 155 | P_out = 155 | 0% Error |
| 4 | Sample Inversion (Input: 255) | P_out = 0 | P_out = 0 | 0% Error |
| 5 | Image Inversion Accuracy | 100% | 100% | 0% Deviation |

Figure 1 Example of image intensity inversion

Result

The image intensity inversion circuit was successfully designed using Verilog RTL and simulated in Xilinx Vivado. The simulation results obtained from Vivado Simulator (XSIM) verified that all 65,536 pixels of the 256 × 256 grayscale image were processed according to the inversion equation (P_{out} = 255 - P_{in}). The results demonstrated that low-intensity pixels were converted into high-intensity pixels and vice versa, confirming the correct operation of the intensity inversion algorithm without arithmetic errors.